# GLIDEPATH — Obstruction Evaluations Capabilities Briefing

Version 2.16.1 | March 2026

## 7. Obstruction Evaluations

### Purpose & Regulatory Basis

The Obstruction Evaluations module implements UFC 3-260-01 (Unified Facilities Criteria) imaginary surface analysis for airfield obstructions. Per DAFMAN 13-204 Vol. 2, airfield managers must evaluate potential obstructions against all imaginary surfaces (Primary, Approach-Departure, Inner Horizontal, Conical, Outer Horizontal, Transitional, Clear Zone, Graded Area) and Accident Potential Zones (APZ I/II per DoD Instruction 4165.57). The module performs real-time geometric calculations against actual runway geometry.

### User Interface

- Evaluation page: app/(app)/obstructions/page.tsx – Interactive Mapbox satellite map for point selection. User clicks to place an obstruction, enters height AGL, and the system evaluates against all surfaces for all runways. Results shown as pass/violation badges per surface.

- History page: app/(app)/obstructions/[id]/page.tsx – Read-only view of a saved evaluation with full surface analysis results, photos, and PDF export.

- Map component: AirfieldMap (dynamic import, no SSR) shows runway outlines, imaginary surface boundaries, and the obstruction point.

- Edit mode: ?edit={id} URL parameter enables editing an existing evaluation.

### Data Model

Primary table: obstruction_evaluations

| Column | Type | Description |
| --- | --- | --- |
| display_id | text | Auto-generated display ID |
| runway_class | enum | B, Army_B |
| object_height_agl | numeric | Object height above ground level (feet) |
| object_distance_ft | numeric | Distance from runway |
| distance_from_centerline_ft | numeric | Lateral offset from centerline |
| object_elevation_msl | numeric | Ground elevation MSL at object location |
| obstruction_top_msl | numeric | Top of object MSL |
| latitude / longitude | numeric | Object coordinates |
| description | text | Object description |
| photo_storage_path | text | JSON array of photo paths or single path |
| results | JSONB | Array of SurfaceResult objects per surface |
| controlling_surface | text | Most restrictive violated surface |
| violated_surfaces | text[] | List of violated surface keys |
| has_violation | boolean | Whether any surface is violated |
| notes | text | Evaluator notes |
| evaluated_by | UUID FK | User who performed evaluation |

### Features

- Multi-runway analysis: evaluateObstructionAllRunways() evaluates the obstruction against all configured runways simultaneously, returning per-runway results.

- 9 imaginary surfaces evaluated: Clear Zone, Graded Area, Primary Surface, Approach-Departure Clearance Surface (50:1 slope), Inner Horizontal (150 ft / 13,120 ft radius), Conical (20:1 slope / 7,000 ft extent), Outer Horizontal (500 ft / 42,250 ft radius), Transitional (7:1 slope), and APZ I/II.

- Runway class support: Surface criteria vary by runway class (B vs Army_B) via getSurfaceCriteria() from lib/calculations/surface-criteria.ts.

- Geodesic geometry engine: lib/calculations/geometry.ts provides Haversine distance (distanceFt()), bearing calculation (bearing()), point offset (offsetPoint()), runway geometry construction (getRunwayGeometry()), and point-to-runway relation (pointToRunwayRelation()).

- Elevation lookup: fetchElevation() queries elevation APIs for ground MSL at the obstruction point.

- Surface identification: identifySurface() determines which imaginary surface(s) the point falls within based on distance from threshold and centerline.

- Per-surface results: Each surface evaluation returns surfaceKey, surfaceName, isWithinBounds, maxAllowableHeightAGL, maxAllowableHeightMSL, obstructionTopMSL, violated (boolean), penetrationFt, ufcReference, and ufcCriteria.

- Photo support: Photos parsed via parsePhotoPaths() which handles JSON arrays, plain URLs, and null values.

- Photo upload: Direct file upload and camera capture.

### Exports

- PDF: generateObstructionPdf() in lib/obstruction-pdf.ts – Portrait letter format with OBSTRUCTION EVALUATION REPORT header, pass/violation status badge, object details (height, elevation, coordinates, description), per-surface analysis results table, UFC references, and photos.

- Email PDF: Via EmailPdfModal.

### Integration Points

- Daily Ops Report: Obstruction evaluations for the selected date are included with evaluator names, violation status, and photos.

- Activity log: All evaluations logged.

- Dashboard: No direct dashboard integration, but violations may trigger discrepancy creation.

### Technical Implementation

| File | Purpose |
| --- | --- |
| app/(app)/obstructions/page.tsx | Interactive evaluation with Mapbox map |
| app/(app)/obstructions/[id]/page.tsx | Evaluation detail/history |
| lib/calculations/obstructions.ts | evaluateObstruction(), evaluateObstructionAllRunways(), identifySurface(), IMAGINARY_SURFACES definitions |
| lib/calculations/geometry.ts | getRunwayGeometry(), pointToRunwayRelation(), distanceFt(), bearing(), offsetPoint(), fetchElevation() |
| lib/calculations/surface-criteria.ts | getSurfaceCriteria() – runway-class-specific surface dimensions |
| lib/supabase/obstructions.ts | CRUD: fetchObstructionEvaluations(), createObstructionEvaluation(), updateObstructionEvaluation(), parsePhotoPaths() |
| lib/obstruction-pdf.ts | generateObstructionPdf() |
| components/obstructions/airfield-map.tsx | Mapbox satellite map with runway overlays |